������  Hardware Scrollies .... Everything You Might Dream Of .... ������

Released   : 17/10/93

Doc & Code : Binary Slave

Last Minute: Living Nature

Binary Slave presents his first source-code to Public Domain.

It mainly consist of the so called Hardware scroll... .

A Hardware scroll is a characteristic of the standard VGA-card.

It does not take hudge CPU time just a few port writes and the rest is up

to another VGA characteristic nl. Vertical retrace which does all the

synchronisation.

I will give you an idea/example how to make a hardware scrolly... .

So you'll be able to make very fast scrollers (I sure hope so) on

your PC even on an XT! Just like in some demos.

All you need is some knowledge about programming pascal/assembly,

graphic environments,image processing.

The code is written in TP & ASM, and may easyly be converted into C

language or similar languages. The complete source is given and you need

at least TP version 6.0.

For the assembly procedures I just saved the DS if it's changed, but if

you want to work 100% properly this doesn't fit. Cause if you don't  ....

God may be with you...lazy me.

You propably need to preserve the following registers:

-  Base pointer  BP.

-  Stack Segment SS.

-  Stack pointer SP.

-  Data Segment  DS.

An asm statement can freely modify these registers:

-  AX,BX,CX,DX

-  SI,DI

-  ES

-  Flags

On entry to the asm statement, the built-in assembler knows only the BP, SP,

SS, and DS registers; it can assume nothing else about register contents.

I've chosen those languages for several reasons.

Turbo Pascal : - one of the most popular languages...

- very easy to read/write/learn and understand some techniques.

ASM : For the fast actions... such page flipping/filling/drawing routines.

It was not the intention to make a super fast scrolling, no, it is

intended to make Hardware scrolling a fact and understandable to all coders,

not just the asm-speed freaks (who already know the hardware scrollies).

The speed of the actual scroll is not so important due to the synchronisations

Learning to make smooth animations is perhaps more important than the speed of

the animation!

Also try not to make too much procedure calls because TP's internal memory

management slows (due to the activation records) down the animation.

The scrolling is performed in tweaked video mode, it is also perfectly

possible that you make your scrollers in normal graphics mode (13H -

320*200*256colors), but I think mode X has got its charms...

First Let me tell you something about your PC...

On the base level of your PC there exist things called I/O ports.

These ports makes it possible to rebuild/change the entire

graphic evironment.

I   Some general functions of the VGA

=================================

I.1 The Graphics Controller

---------------------------

The graphics controller is a VLSI circuit that resides in the data path

between the processor and the display memory. In its default state, the

graphics controller is transparent. As a programmer you should be able to

modify this port in order to let it assist in drawing operations. Like

logically combine,rotate processor data with the display memory data

(PROCESSOR READ LATCHES, which can be verry usefull...).

Color compare functions (rep scasb ex.sprites),Data serializer,Logical Unit.

I.2 The Crt Controller Registers

---------------------------------

The CRT controller controls the crt (Cathode Ray Tube) by generating the

syncing and blanking signals that define the display raster.

It displays colors that are generated by a beam of electrons that strike

the phosphorus coating on the back of the crt screen and cause it to glow.

The electron beam is swept across the display screen from the left to the

right in a series of horizontal lines. At the same time, it's intensity is

modulated to produce display patterns. The electron beam must continiously

redraw the pattern on the screen 50,60,70 or more times a second, depending

on the display used (=REFRESH TIME). The sweep pattern of the electron

beam is called RASTER LINES.

The beam begins in the upper left corner of the display and sweeps right.

When it reaches the right edge of the screen, the beam is shut off (HORIZONTAL

BLANKING) and then rapidly brought back to the left edge of the screen

(HORIZONTAL RETRACE) to begin the next horizontal scan just below the previous

one. After all the horizontal scans have been completed (HORIZONTAL TOTAL),

the electron beam will end up in the lower right corner of the screen.

At this point the beam is shut off once again (VERTIKAL BLANKING) and then

rapidly brought back up to the upper left corner (VERTIKAL RETRACE) so the

next raster can begin.

This last effect is normally very synchronic on all VGA-cards, so this

effect should be very good to synchronize all our animation...Ok?

I.3 Display Memory

------------------

The nowadays VGA contains up to 256Kb or more display memory, divided into

four independent 64Kb sections of memory! These are called color planes.

These color planes all reside in the same processor memory space.

Which color planes are being written to or read from at any time is

determined by several I/O ports.

I.4 Attribute Controller

------------------------

The attribute controller determines wich color will be displayed for both text

and graphic mode. The heart of the controller is the color look-up table that

translate four bit information from display memory into 6 bit color information

mation (EGA) or 12 bit color information (VGA).

I.5 Sequencer

-------------

The sequencer controls the sequencing of all functions. It generates the dot

and character clocks that control display refresh timing. It controls the

timing of display memory read and write cycles, and generate wait-states

if necessary.

II  The VGA Mode

============

The standard (MCGA) or bios 13H mode has a resolution of 320*200 and

256 colors. You can call this mode by a simple BIOS-call. The BIOS takes

care of all the additional settings, no prob.

To get into tweaked video mode which has a resolution of 320*200*4 and

256 colors, we have to do some old tricks.

-->"unchain" the vga mode, disable the "odd/even adressing".<--

These setting enable us to use 256kb of display memory.

Yes, 4 * 64Kb color planes are available, this results in a smooth page

flipping animation scenes...(if handled properly ofcoz).

As you all know the VGA memory adress area starts from 0A000:0000H.

In tweaked mode we have:

0A000:0000 equals 1th pixel + 2nd pixel + 3th pixel + 4th pixel!

0A000:0001 equals 5th pixel + 6th pixel + 7th pixel + 8th pixel!

etc...

The only thing distinguishing the pixels is the memory adress and the color

planes which they are in.

As we know we have 4 planes ... 4 pixel

0A000:0000  1th plane ....  1th pixel

0A000:0000  2nd plane ....  2nd pixel

0A000:0000  3th plane ....  3th pixel

0A000:0000  4th plane ....  4th pixel

0A000:0001  1th plane ....  5th pixel

0A000:0001  2nd plane ....  6th pixel

0A000:0001  3th plane ....  7th pixel

0A000:0001  4th plane ....  8th pixel

etc etc... ok?

Remark:  In our case this means that the physical screen is only 80 memory

adress spaces (pixels) in width!! No longer 320!!

200 in height remains the same.

III The used i/o ports:

===================

III.1 Plane reading/writing

---------------------------

The Graphics controller: port 3CEH

index  00  : Set/reset register

01  : Set/reset enable register

02  : Color compare register

03  : Data rotate & Function Select

04  : Read plane Select register

05  : Mode register

06  : Miscellaneous register

07  : Color Don't care register

08  : Bit mask register...

Writing on the screen...

Write to plane ... plane enable (port 3CEH index 01).

p.e.: mov   DX,3ceH    ; select port 3CEH

mov   al,01      ; select index 01

out   dx,al      ; write selections to port

inc   dx

d0 d1 d2 d3  *  *  *  *

port 3CEH index 1              �����������������������ͼ

0  1  2  3  4  5  6  7

* = Not used by plane switching.

d0=1 : plane 0 write enbale

d0=0 : plane 0 write disabled!

d1...  plane 1.

d2...  plane 2.

d3...  plane 3.

Yes, it is possible to write with one movsb to write 4 pixels at

once... (yes even eight if you enable double word mode).

Such routines are used to clear verry fast the screen in graphics

mode just imagine 4 64Kb display memory screen cleared at the

speed of clearing 2 screens (4K) in text mode !!!

OR

mov   al,00 --> in  al,dx

and al,11110000B force all planes on write disabled.

or  al,1   force bit 0 on 1 to make plane 0

write enable..

out   dx,al    ; write changes to port.

For planes 1..3 the same code san be used.

plane 1: mov al,02H

plane 2: mov al,04H

plane 3: mov al,08H

all planes: mov al,0FH

Reading from memory/screen:

Read plane select register... (port 3CEH index 04)

This one differs from the plane enable :-)

d0 d1  *  *  *  *  *  *

port 3CEH index 04             �����������������������ͼ

0  1  2  3  4  5  6  7

* = Not used by plane switching.

d0&d1=00 : plane 0 read enbale.

01 : plane 1 read enable.

10 : plane 2 read enable.

11 : plane 3 read enbale.

you are not allowed to read from more than one plane at a time since

this would not be meaningfull.

So, in this way you are able to read from the screen.

III.2  The Logical Screen Width

-------------------------------

In graphics mode, it defines the logical distance, in either 16 bit

memory words or 32 bit double words between successive scan lines.

In all standard operating modes,the logical screen width is equal to

the physical screen width.

You can change the width of the screen to make you screen wider so you

can update you scrolly (animation) out of the screen so nobody will

see it!!

In the example we use 40*4=160 words this means 320 bytes in width.

Actually *4 =1280 bytes wide!!!!

You can change all these settings for example zooming or the hardware

scroll ( Panning) page flipping animation.

You can alther the screen width with the CRT controller... .

PORT 3B4/3D4H

-------------

index 00H  Horizontal total           0CH  Start adress high

01H  Horizontal display end     0DH  Start adress low

02H  Start horizontal blank     0EH  Cursor location high

03H  End horizontal blank       0FH  Cursor location low

04H  Start horizontal retrace   10H  Vertikal retrace start

05H  End horizontal retrace     11H  Vertikal Retrace end

06H  Vertical total             12H  Vertikal Display end

07H  Overflow                   13H  Logical Screen width

08H  Preset row scan            14H  Underline location

09H  Maximum scan line          15H  Start Vertikal Blank

0AH  Cursor Start               16H  End Vertikal Blank

0BH  Cursor end                 17H  Mode Control register

18H  Line Compare register

Handle these ports with care because some of them can damage your

monitor screen!!!

Well the index we are intersted in is 13H:

ex. in  TP:

port[$3d4]:=$13    ;select port 3D4H index 13H = logical screen width.

port[$3d5]:=.....  length of the screen ... in our case 40*4 =160 words.

IV   The Actual Hardware Scroll

==========================

a) Byte Panning

---------------

If we take a look at the CRT controller port 3D4H we see the index

0CH and 0DH.

The start adress high and Start adress low... .

This is a 16 bit read/write register that defines the adress in

display memory of the data that will be displayed in the upper left

corner of the screen (starting position).

This register can be used to byte-panning an image on the screen or to do

the page-flipping.

1 word equals    High  Low

���������ͼ

0CH = Start addres (high byte).

0DH = Start addres (low  byte).

b) But Panning

--------------

For smooth scrolling in text mode for example we can add the index 08H

Preset row scan, so that characters can be scrolled up or down one pixel

at a time! See also Horizontal Pel Pan register.

d0 d1 d2 d3 d4  *  *  *

port 3CEH index 08             �����������������������ͼ

0  1  2  3  4  5  6  7

* = Not used by panning.

d4-d0: Preset Row scan for panning one pixel at a time.

Note we wont use this because this realy isn't nessecary in

graphics mode to perform smooth panning.

Horizontal Pel Pan Register

---------------------------

First of all this register is situated at port 3C0H (= Attribute Contr-

oller Register.)

PORT 3C0H:

----------

register  00H Color palette Register 0   0AH Color palete Register 10

01H Color palette Register 1   0BH Color palete Register 11

02H Color palette Register 2   0CH Color palete Register 12

03H Color palette Register 3   0DH Color palete Register 13

04H Color palette Register 4   0EH Color palete Register 14

05H Color palette Register 5   0FH Color palete Register 15

06H Color palette Register 6   10H Mode Control Register

07H Color palette Register 7   11H Screen Border color

08H Color palette Register 8   12H Color plane enable register

09H Color palette Register 9   13H Horizontal Panning regsiter

14H Color Select register

d0 d1 d2 d3  *  *  *  *

port 3C0H index 13H            �����������������������ͼ

0  1  2  3  4  5  6  7

* = Reserved.

d3-d0: Allows the display to be shifted horizontally one pixel at

a time. Up to 16 pixels the rest is performed by the start

address register increasing 2 times or 2 words. or 2*8=16!

This might all sound pretty confusing but the following pic should

clear things up...

ex. in text mode: 1 ��������

2 ��������

3 ��������

4 ��������

5 ����   �= left upper corner.

6 ��������   Preset row Scan = 5   -- VERTICAL.

7 ��������   Horiz. Pel Pan  = 3   -- HORIZONTAL.

8 ��������   These pixel per pixel.

12345678

While Start adress register byte per

byte... .

if � (the upper left corner) reaches the 8,8 then it is time to clear

the preset row scan and the Hor.pel pan reg. and increase the Start

adress register. OK?

Note: If you make a split screen you are able to put an image on the screen

and let the Hardware scrolly scroll on top above the split screen.

The linecounter ignores the split screen and so the VGA is fooled.

So the panning has now effect on the splitscreen!

Just like in the MN2-Dentro...

The Line compare register enables us to use a split screen.

The line compare register has several overflow bits seperated

in different ports, what a genius plan!!

Just put the correct value in the line compare and off we go... .

Upper left corner :    --> ������������������������Ŀ

Start adress register      �-----                   �

�----------              �

�--------------          �

�                        �

Line compare register: --->�========================�

Begin of the display       �---- ---- ---- ---- ----�

memory.                    �---- ---- ---- ---- ----�

�--------------          �

�---- ---- .             �

�                        �

�                        �

��������������������������

In the MN2-dentro I used the upper part of the screen as hardware

scrolly. The scrolly is on the buttom of each page .

So I could leave the big pix on the top of page 2 and made

it pan the way you saw it.

Well if you find something interesting, or if you make some scrollies yourself

with this source, I would be pleased if you upload them at MN2.

Also if you have an idea/comment please upload it at MN2 in the

Binary Slave conference...

Still having trouble?

Leave a msg at MN2 and I'll see what I can do...

Files:

------

- Cijf.pcx

- Multa-u.pcx

- Multv-.pcx        {GFX files made by Wolf. My personal gfx man!}

- Example.hdw       {"Modulated" scroll text}

- Example.txt       {Scroll text}

- Explain.doc       {This file}

- Hscroll.exe       {Compiled example file}

- Hscroll.pas       {Example source}

- Meat.exe          {MN2 publicement file}

- Meat.pas          {Source from this file}

- Txt2scr.exe       {Converts example.txt 2 example.hdw ascii# 33 - 100}

- Txt2scr.pas       {Source for this file

Note: I don't have the official documents of the Vga, only some books.

I'm not responsable for any faults in these docs!

Take care with some ports because they might cause damage!

The only method to learn such things is trail and error!

Be sure you get the whole and correct source and D/L it from MN2!

| MultiNet 2: +32 (0)50/36.33.96 [HST 24/24]             |

| FidoNet   : 2:291/1800.12 [Bart Vanderostyne]          |

| F A X     : +32 (0)50/34.73.79 [Write to Binary Slave] |

Hi's and Ho's  to - D.H.

-------------     - Tasmaniac

- Shadowlord

- Overlord 0f House

- Living Nature

- El Presidento

Greetings to all wich I humbly forgot on the MN2 greetingslist...

<EOF>